Absolutely. I read the uploaded “Long-Horizon Vibration Spectrogram Forecasting with Convolutional LSTM and Iterative Next-Window Rollout” chapter. It is a standalone methodological chapter describing a complete pipeline for predicting future vibration spectrograms over long time horizons using STFT + ConvLSTM + iterative feedback.

Below is a detailed explanation of what the entire chapter is doing, why each step is necessary, and how the different parts connect.

# Chapter Overview

The central idea of the chapter is very simple:

Do not try to predict the vibration spectrogram several hours or days into the future in one prediction. Instead, learn to predict only the next spectrogram, then use that prediction as the input for the following prediction, and repeat the process.

The chapter therefore converts a difficult long-term forecasting problem into a sequence of much smaller one-step forecasting problems.

The complete pipeline is:

Raw vibration signal → major windows → STFT → log-magnitude spectrogram → recent-spectrogram context → ConvLSTM → next spectrogram → feedback → repeat → long-horizon forecast

The chapter explicitly describes this as an eight-step process.

# 1. Motivation and Scope

The problem begins with an industrial machine producing vibration measurements continuously.

Suppose the machine produces a vibration signal:

x[n]x[n]

at a sampling frequency of:

fs=1200 Hz.f_s = 1200\text{ Hz}.

The goal is not merely to determine what the machine is doing right now. The objective is to predict how its frequency-domain vibration characteristics will evolve in the future.

This is important because machine faults often appear as changes in frequency components.

For example, a bearing problem might produce:

- increasing energy around a characteristic frequency,

- additional harmonics,

- sidebands,

- increasing broadband vibration,

- changes in existing spectral ridges.

Looking at the raw waveform makes these changes difficult to interpret. A spectrogram provides a much more useful representation because it shows how frequency content changes over time.

The chapter therefore converts vibration signals into spectrograms before applying the neural network.

# 2. Why Not Predict Several Days Directly?

One of the most important arguments in the chapter concerns the difficulty of direct long-horizon prediction.

Imagine that each major vibration window represents two seconds.

A one-day prediction would require:

24×36002=43,200\frac{24\times3600}{2}=43,200

future windows.

The chapter therefore argues that directly asking a neural network to produce the spectrogram 43,200 windows into the future is poorly posed. Operating conditions can change, loads can change, and uncertainty accumulates over long periods.

Instead, the model learns:

St−k+1,...,St→St+1S_{t-k+1},...,S_t \rightarrow S_{t+1}

and then:

St−k+2,...,St,S^t+1→S^t+2.S_{t-k+2},...,S_t,\hat S_{t+1} \rightarrow \hat S_{t+2}.

This continues repeatedly.

Therefore:

long horizon=many one-step predictions.\text{long horizon} = \text{many one-step predictions}.

This is the fundamental methodological principle of the chapter.

# 3. The Eight-Step Architecture

The entire methodology can be understood as:

Raw vibration signal

│

▼

┌───────────────────┐

│ 1. Major Windows  │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ 2. Subwindow STFT │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ 3. Spectrogram    │

│    construction   │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ 4. Context Buffer │

│  recent spectra   │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ 5. ConvLSTM       │

│    predictor      │

└─────────┬─────────┘

│

▼

Ŝ(t+1)

│

▼

┌───────────────────┐

│ 6. One-step       │

│    training       │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ 7. Feedback and   │

│    iteration      │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ 8. Long-horizon   │

│    forecast       │

└───────────────────┘

The important point is that Step 7 turns a one-step model into a long-horizon forecasting system.

# 4. Step 1 — Creating Major Vibration Windows

The continuous vibration signal is first divided into major windows.

The chapter uses:

fs=1200 Hzf_s=1200\text{ Hz}

and:

Tw=2 s.T_w=2\text{ s}.

Therefore, every major window contains:

Nw=fsTwN_w=f_sT_w

samples.

Thus:

Nw=1200×2=2400.N_w=1200\times2=2400.

So each major window contains 2,400 vibration samples.

Conceptually:

Continuous vibration

─────────────────────────────────────────────────────>

|------ 2 s ------|------ 2 s ------|------ 2 s ------|

x(t)               x(t+1)             x(t+2)

The chapter uses non-overlapping major windows.

That is important because each window has a clear temporal meaning.

If:

xtx_t

represents one window, then:

xt+1x_{t+1}

represents the immediately following window.

This gives a clean training relationship:

xt→xt+1.x_t\rightarrow x_{t+1}.

# 5. Step 2 — Subwindows and STFT

A 2-second vibration window is still too large to represent with a single FFT if we want to see how its frequency content evolves internally.

Therefore, each major window is divided into smaller overlapping subwindows.

The default parameters are:

Ts=0.25 sT_s=0.25\text{ s}

and therefore:

L=fsTsL=f_sT_s

giving:

L=1200(0.25)=300.L=1200(0.25)=300.

The STFT hop is:

Hstft=150.H_{stft}=150.

So the subwindows overlap.

Conceptually:

2-second major window

|------------------------------------------------|

|------|

|------|

|------|

|------|

|------|

|------|

...

This produces:

M=15M=15

STFT frames for the default configuration. The frequency resolution is:

Δf=fsL\Delta f=\frac{f_s}{L}

which gives:

Δf=1200300=4 Hz.\Delta f=\frac{1200}{300}=4\text{ Hz}.

The chapter specifically chooses STFT because its uniform frequency bins provide an interpretable frequency representation for shaft orders and defect frequencies.

# 6. Hann Window

Before calculating each FFT, the signal segment is multiplied by a Hann window:

w[n]=0.5−0.5cos⁡(2πnL−1).w[n] = 0.5 - 0.5\cos \left( \frac{2\pi n}{L-1} \right).

This reduces discontinuities at the boundaries of each FFT segment.

The resulting STFT is:

X[m,k]=∑n=0L−1x[n+mHstft]w[n]e−j2πkn/L.X[m,k] = \sum_{n=0}^{L-1} x[n+mH_{stft}] w[n] e^{-j2\pi kn/L}.

The result contains information in two dimensions:

- time/frame index mm,

- frequency-bin index kk.

Thus the STFT transforms:

time-domain vibration\text{time-domain vibration}

into:

time-frequency representation.\text{time-frequency representation}.

# 7. Step 3 — Constructing the Spectrogram

The chapter does not directly use the complex STFT.

Instead, it uses the magnitude:

∣X[m,k]∣.|X[m,k]|.

Then it applies logarithmic compression:

St[m,k]=log⁡(1+∣X[m,k]∣).S_t[m,k] = \log(1+|X[m,k]|).

This creates the log-magnitude spectrogram.

The logarithm is important because vibration spectra can have a very large dynamic range.

Without compression, a very strong frequency component could dominate the representation.

The transformation:

log⁡(1+∣X∣)\log(1+|X|)

helps:

- compress large amplitudes,

- stabilize neural-network training,

- prevent problems with log⁡(0)\log(0),

- retain spectral-energy information,

- avoid requiring the network to predict unstable phase information.

The resulting spectrogram has shape:

St∈RM×F.S_t\in\mathbb{R}^{M\times F}.

For the default parameters:

M=15M=15

and:

F=129F=129

because frequencies are retained up to:

fmax⁡=512 Hz.f_{\max}=512\text{ Hz}.

Therefore:

St∈R15×129.S_t\in\mathbb{R}^{15\times129}.

For multiple sensors, the shape becomes:

St∈RC×M×F.S_t\in\mathbb{R}^{C\times M\times F}.

# 8. Why the Spectrogram Is the Input to the Neural Network

This is an important conceptual point.

The network is not directly forecasting:

x[n].x[n].

It is forecasting:

St.S_t.

That means the model is learning the evolution of frequency-domain vibration characteristics.

For example:

Time

↓

500 Hz |       █

|       █

400 Hz |      ██

|      ██

300 Hz |    ███

|    ███

200 Hz |  ████

|  ████

100 Hz | █████

+----------------→

Time

A fault-related frequency ridge may become brighter or shift over successive spectrograms.

The neural network therefore learns patterns such as:

St−2→St−1→St→St+1.S_{t-2}\rightarrow S_{t-1}\rightarrow S_t \rightarrow S_{t+1}.

# 9. Step 4 — Context Buffer

A single spectrogram only tells us about the current vibration condition.

The chapter therefore uses several recent spectrograms as context.

The default context length is:

k=3.k=3.

Thus:

Ut=(St−2,St−1,St).U_t= (S_{t-2},S_{t-1},S_t).

The context tensor has the shape:

Ut∈Rk×C×M×F.U_t\in \mathbb{R}^{k\times C\times M\times F}.

For one sensor:

Ut∈R3×1×15×129.U_t\in \mathbb{R}^{3\times1\times15\times129}.

This gives the model a short history.

For example:

Older                         Current

S(t-2)       S(t-1)       S(t)

│             │            │

▼             ▼            ▼

┌───────┐    ┌───────┐    ┌───────┐

│Spec.  │    │Spec.  │    │Spec.  │

└───────┘    └───────┘    └───────┘

\           |           /

\          |          /

└─────────┴──────────┘

│

▼

ConvLSTM

│

▼

Ŝ(t+1)

Why is this better than using only StS_t?

Because the network can observe trends.

For example:

St−2<St−1<StS_{t-2} < S_{t-1} < S_t

in terms of energy around a particular frequency.

The network can learn that the component is increasing and potentially predict:

St+1S_{t+1}

with stronger energy in that region.

The chapter identifies k=3k=3 as a practical compromise between historical information, computational cost, and latency.

# 10. Step 5 — Why ConvLSTM?

This is the central neural-network component.

The chapter deliberately chooses ConvLSTM rather than a collection of alternative neural architectures.

The reason is that a spectrogram is simultaneously:

- spatial,

- temporal,

- frequency-dependent.

The two dimensions of a spectrogram correspond roughly to:

time×frequency.\text{time} \times \text{frequency}.

Therefore, the model needs to understand local structures within the spectrogram while also remembering how those structures evolve between consecutive spectrograms.

ConvLSTM is designed for this.

# 11. Difference Between LSTM and ConvLSTM

A traditional LSTM treats its input as vectors.

A conventional LSTM gate might be written:

it=σ(Wxixt+Whiht−1+bi).i_t = \sigma (W_{xi}x_t + W_{hi}h_{t-1} + b_i).

If the spectrogram is flattened into a vector, the spatial structure disappears.

For example:

Original spectrogram

frequency

↑

│ ██

│  ██

│   ██

│    ███

└──────────→ time

After flattening:

[0.02, 0.01, 0.08, 0.91, ...]

The network no longer explicitly knows that neighboring frequency bins were neighbors.

ConvLSTM instead replaces matrix multiplication with convolution:

Wxixt→Wxi∗Xt.W_{xi}x_t \rightarrow W_{xi}*X_t.

Therefore, local structures remain spatially organized.

This is particularly useful for vibration because harmonic lines and defect-frequency ridges are local structures in the time-frequency plane.

# 12. ConvLSTM Gate Mechanism

The ConvLSTM contains four major components:

- input gate,

- forget gate,

- output gate,

- candidate state.

The equations are:

it=σ(Wxi∗Xt+Whi∗Ht−1+bi)i_t = \sigma (W_{xi}*X_t+ W_{hi}*H_{t-1}+b_i) ft=σ(Wxf∗Xt+Whf∗Ht−1+bf)f_t = \sigma (W_{xf}*X_t+ W_{hf}*H_{t-1}+b_f) ot=σ(Wxo∗Xt+Who∗Ht−1+bo)o_t = \sigma (W_{xo}*X_t+ W_{ho}*H_{t-1}+b_o) gt=tanh⁡(Wxg∗Xt+Whg∗Ht−1+bg).g_t = \tanh (W_{xg}*X_t+ W_{hg}*H_{t-1}+b_g).

The cell state is then updated using:

Ct=ft⊙Ct−1+it⊙gtC_t = f_t\odot C_{t-1} + i_t\odot g_t

and the hidden state is:

Ht=ot⊙tanh⁡(Ct).H_t = o_t\odot\tanh(C_t).

These equations are explicitly defined in the chapter.

# 13. Physical Interpretation of the Gates

The chapter provides a useful interpretation specifically for vibration spectrograms.

### Forget gate

ftf_t

determines which information from the previous state should remain.

For example, if a strong shaft harmonic has persisted for several windows, the forget gate can preserve that information.

### Input gate

iti_t

controls what new information should be written into memory.

If a defect frequency starts becoming stronger, the input gate can allow that information into the memory.

### Candidate

gtg_t

represents candidate new information.

### Output gate

oto_t

controls what information from the internal memory should be exposed to the next layer.

Therefore, ConvLSTM effectively maintains a time-frequency memory.

# 14. The Actual ConvLSTM Network

The chapter uses a concrete architecture:

Input

(3, C, 15, 129)

│

▼

Conv Stem

3×3

32 channels

│

▼

ConvLSTM-1

64 channels

3×3

│

▼

ConvLSTM-2

64 channels

│

▼

Conv Decoder

32 → C

│

▼

Predicted spectrogram

(C, 15, 129)

The complete network is represented mathematically as:

S^t+1=Dψ(ConvLSTM⁡ϕ(Ut)).\hat S_{t+1} = D_\psi \left( \operatorname{ConvLSTM}_\phi(U_t) \right).

# 15. Why the Convolutional Stem?

Before the recurrent layers, the chapter uses a convolutional stem.

Its purpose is to extract local patterns.

For example:

- harmonic lines,

- spectral edges,

- bursts,

- localized frequency structures.

The chapter explains that the raw log-spectrogram may contain noise mixed with useful structures. The convolutional stem therefore creates more useful feature maps before the recurrent memory processes them.

So conceptually:

St→local spectral features→temporal memory.S_t \rightarrow \text{local spectral features} \rightarrow \text{temporal memory}.

# 16. Why Two ConvLSTM Layers?

The architecture contains two ConvLSTM layers.

The first layer learns relatively direct spatiotemporal patterns.

The second layer can build deeper temporal abstractions.

The chapter describes two layers as a compromise:

- one layer may underfit cross-window trends,

- substantially more layers increase complexity and can contribute to rollout drift.

This is why the chapter chooses two recurrent layers as its default configuration.

# 17. Sequence-to-One Prediction

An important detail is that this is sequence-to-one.

The input contains multiple spectrograms:

St−2,St−1,St.S_{t-2},S_{t-1},S_t.

But the output is only:

S^t+1.\hat S_{t+1}.

So:

S(t-2) ─┐

│

S(t-1) ─┼──► ConvLSTM ───► Ŝ(t+1)

│

S(t)   ─┘

This is intentional.

The model does not attempt to predict:

St+1,St+2,St+3,...,St+HS_{t+1},S_{t+2},S_{t+3},...,S_{t+H}

in a single call.

Instead, it predicts exactly one future window.

That makes training and deployment use the same fundamental interface.

# 18. Step 6 — Training the Model

The training dataset consists of consecutive examples:

(Ut,St+1).(U_t,S_{t+1}).

For example:

Input                         Target

S(t-2) ─┐

S(t-1) ─┼──► ConvLSTM ───► Ŝ(t+1)

S(t)   ─┘                     │

▼

compare

│

▼

S(t+1)

The network predicts:

S^t+1=fθ(Ut).\hat S_{t+1} = f_\theta(U_t).

The prediction is compared with the real next spectrogram.

# 19. Spectrogram Normalization

Before training, the chapter normalizes spectrogram values using the training-set mean and standard deviation:

S~=S−μσ.\tilde S = \frac{S-\mu}{\sigma}.

This prevents large numerical differences in the input representation from making optimization unnecessarily difficult.

Importantly, the normalization parameters should be determined from the training data, rather than independently calculated from future test information.

# 20. MSE Loss

The main loss is mean squared error:

L=∥fθ(Ut)−St+1∥F2.L = \left\| f_\theta(U_t)-S_{t+1} \right\|_F^2.

The idea is to make the predicted spectrogram numerically close to the real spectrogram.

Conceptually:

Predicted spectrogram

│

│

▼

┌─────────┐

│   MSE   │

└────┬────┘

│

▼

Real spectrogram

The chapter chooses MSE because it provides a stable objective for predicting the conditional mean of the next spectrogram and is also useful for subsequent residual monitoring.

# 21. Optional Band-Weighted Loss

The chapter also defines an optional frequency-band weighting term.

The motivation is that some frequency regions may be more diagnostically important than others.

The extended loss is:

L=LMSE+γ∑k∈Bλk∥fθ(Ut)−St+1∥22.L = L_{\text{MSE}} + \gamma \sum_{k\in B} \lambda_k \left\| f_\theta(U_t)-S_{t+1} \right\|_2^2.

Here:

- BB = selected diagnostic frequency bins,

- λk\lambda_k = importance weight,

- γ\gamma = strength of the additional term.

This allows the model to place more emphasis on frequency regions that are particularly relevant to machine behavior.

# 22. Teacher Forcing

Training initially uses teacher forcing.

That means that when training the model, the context consists of real spectrograms.

For example:

Real S(t-2)

Real S(t-1)

Real S(t)

│

▼

ConvLSTM

│

▼

Predicted S(t+1)

The advantage is that the network first learns the basic one-step dynamics under relatively clean conditions.

The chapter recommends this because it stabilizes initial training.

# 23. The Train/Deployment Mismatch

There is, however, an important problem.

During training:

input history=real history.\text{input history}=\text{real history}.

During long-horizon deployment:

input history=real + predicted history.\text{input history} = \text{real + predicted history}.

Eventually, the model may receive only its own predictions.

This creates a mismatch.

For example:

Training:

Real → Real → Real → Prediction

Deployment:

Real → Real → Prediction

↓

Prediction

↓

Prediction

↓

Prediction

Small errors can therefore accumulate.

# 24. Scheduled Sampling

The chapter proposes scheduled sampling as an optional solution.

At training step nn, the probability of using the true spectrogram is:

pn=max⁡(pmin⁡,p0ηn).p_n = \max \left( p_{\min}, p_0\eta^n \right).

The default values are:

p0=1,p_0=1, η=0.995,\eta=0.995, pmin⁡=0.7.p_{\min}=0.7.

Thus the network gradually becomes accustomed to its own predictions entering the context.

The purpose is not to change the fundamental model.

It is to make the model more robust when it eventually operates in feedback mode.

# 25. Step 7 — Iterative Feedback

This is arguably the most important part of the chapter.

Suppose the initial context is:

Ut=(St−2,St−1,St).U_t= (S_{t-2},S_{t-1},S_t).

The model predicts:

S^t+1=fθ(Ut).\hat S_{t+1}=f_\theta(U_t).

Then the predicted spectrogram is inserted into the context.

The new context becomes:

Ut+1=(St−1,St,S^t+1).U_{t+1} = (S_{t-1},S_t,\hat S_{t+1}).

Then:

S^t+2=fθ(Ut+1).\hat S_{t+2} = f_\theta(U_{t+1}).

The process continues.

# 26. Exact Feedback Example

With k=3k=3:

### Initial

(St−2,St−1,St)(S_{t-2},S_{t-1},S_t)

### Prediction 1

S^t+1\hat S_{t+1}

Buffer becomes:

(St−1,St,S^t+1)(S_{t-1},S_t,\hat S_{t+1})

### Prediction 2

S^t+2\hat S_{t+2}

Buffer becomes:

(St,S^t+1,S^t+2)(S_t,\hat S_{t+1},\hat S_{t+2})

### Prediction 3

S^t+3\hat S_{t+3}

Buffer becomes:

(S^t+1,S^t+2,S^t+3).(\hat S_{t+1},\hat S_{t+2},\hat S_{t+3}).

At this point, the entire context is synthetic.

This exact transition is described in the chapter's worked feedback example.

# 27. Why FIFO?

The context buffer operates like a FIFO queue:

First In → First Out

When a new prediction arrives:

- the oldest spectrogram is removed,

- the new prediction is appended.

For example:

Before:

[S(t-1), S(t), Ŝ(t+1)]

↓ new prediction

After:

[S(t), Ŝ(t+1), Ŝ(t+2)]

This makes the forecasting system behave like an online process in which time continuously moves forward.

# 28. How Can This Produce a One-Day Forecast?

This is where the methodology becomes interesting.

Each major window is:

Tw=2 s.T_w=2\text{ s}.

Therefore:

### One minute

H=602=30.H=\frac{60}{2}=30.

### One hour

H=36002=1800.H=\frac{3600}{2}=1800.

### Twelve hours

H=432002=21600.H=\frac{43200}{2}=21600.

### One day

H=864002=43200.H=\frac{86400}{2}=43200.

These values are explicitly provided in the chapter.

Therefore, a one-day forecast does not mean that the network has 43,200 output neurons.

It means:

43,200 repeated one-step predictions\boxed{ 43,200\text{ repeated one-step predictions} }

using the same trained network.

# 29. The Key Distinction: Model vs Rollout

This distinction is essential if you are explaining this methodology academically.

The model is:

fθ:Ut→S^t+1.f_\theta: U_t\rightarrow\hat S_{t+1}.

It only predicts one step.

The rollout mechanism is:

Ut+1=FIFO⁡(Ut,S^t+1).U_{t+1} = \operatorname{FIFO} (U_t,\hat S_{t+1}).

Then the same model is called again.

Therefore:

fθ(Ut)→S^t+1→fθ(Ut+1)→S^t+2→⋯f_\theta(U_t) \rightarrow \hat S_{t+1} \rightarrow f_\theta(U_{t+1}) \rightarrow \hat S_{t+2} \rightarrow \cdots

The long horizon comes from iteration, not from a giant neural-network output layer.

# 30. Open-Loop Drift

The chapter is very explicit that iterative prediction has a limitation:

errors accumulate.

Suppose the first prediction has a small error:

S^t+1=St+1+ϵ1.\hat S_{t+1}=S_{t+1}+\epsilon_1.

That predicted spectrogram is then used as an input.

The next prediction may therefore contain another error:

S^t+2=St+2+ϵ2.\hat S_{t+2} = S_{t+2} + \epsilon_2.

As predictions continue:

ϵ1,ϵ2,ϵ3,…\epsilon_1,\epsilon_2,\epsilon_3,\ldots

can accumulate.

This phenomenon is called rollout drift.

The chapter explicitly identifies open-loop drift as an honest limitation of long-horizon forecasting.

# 31. Why Re-Anchoring Is Important

The proposed practical solution is to periodically replace predictions with real measurements.

Suppose new real data arrive.

Instead of continuing:

Prediction

↓

Prediction

↓

Prediction

↓

Prediction

the system can periodically do:

Real measurement

↓

Refresh context

↓

ConvLSTM

↓

Prediction

↓

Prediction

↓

Real measurement

↓

Refresh

This is called a hybrid mode.

The chapter argues that this is more realistic for continuous industrial monitoring than relying entirely on an open-loop forecast for an entire day.

# 32. Three Drift-Control Strategies

The chapter identifies three controls.

### 1. Re-anchoring

Periodically reset the prediction buffer using measured spectrograms.

Default:

R=1R=1

while streaming.

### 2. Prediction smoothing

A mild frequency-domain smoothing operation can optionally be applied before feeding a prediction back into the system.

The purpose is to reduce small artifacts that could otherwise become amplified during repeated feedback.

### 3. Horizon-wise validation

Instead of evaluating only one-step prediction:

MSE(1),MSE(1),

evaluate:

MSE(1),MSE(2),...,MSE(h).MSE(1),MSE(2),...,MSE(h).

A model that performs well at one step but rapidly deteriorates during rollout should therefore be identified.

# 33. Step 8 — Long-Horizon Outputs

After HH iterations, the system produces:

(S^t+1,S^t+2,...,S^t+H).\left( \hat S_{t+1}, \hat S_{t+2}, ..., \hat S_{t+H} \right).

This is the final long-horizon forecast.

The output is therefore not just one image.

It is a sequence of predicted spectrograms.

Conceptually:

Current

│

▼

Ŝ(t+1)

│

▼

Ŝ(t+2)

│

▼

Ŝ(t+3)

│

▼

Ŝ(t+4)

│

▼

...

│

▼

Ŝ(t+H)

This sequence describes the predicted evolution of the machine's spectral behavior.

# 34. Monitoring Signals

The chapter proposes two useful quantities.

## Actual prediction residual

When the true future becomes available:

at+h=∥St+h−S^t+h∥F.a_{t+h} = \left\| S_{t+h}-\hat S_{t+h} \right\|_F.

This measures the discrepancy between predicted and observed spectrograms.

A large residual can indicate that the actual machine behavior differs from the learned dynamics.

# 35. Predicted Spectral Change

The second quantity is:

et+h=∥S^t+h−S^t+h−1∥F.e_{t+h} = \left\| \hat S_{t+h} - \hat S_{t+h-1} \right\|_F.

This measures how rapidly the predicted spectrum is changing.

The interesting aspect is that this quantity can be calculated before the future measurement becomes available.

Therefore:

- aa compares prediction with reality,

- ee examines predicted future evolution.

A sudden increase in predicted change may indicate unstable predicted evolution or a rapidly changing predicted spectral pattern.

# 36. Evaluation

The chapter emphasizes that evaluating only one-step accuracy is insufficient.

The primary metrics are:

MSE(h)=1MF∑m,k(S^t+h[m,k]−St+h[m,k])2MSE(h) = \frac{1}{MF} \sum_{m,k} \left( \hat S_{t+h}[m,k] - S_{t+h}[m,k] \right)^2

and:

MAE(h)=1MF∑m,k∣S^t+h[m,k]−St+h[m,k]∣.MAE(h) = \frac{1}{MF} \sum_{m,k} \left| \hat S_{t+h}[m,k] - S_{t+h}[m,k] \right|.

The critical recommendation is to plot:

MSE(h)MSE(h)

against:

h.h.

For example:

MSE

↑

│                         *

│                    *

│               *

│           *

│       *

│    *

│  *

│ *

└────────────────────────────→

Horizon h

This shows how prediction quality changes as the forecast moves further into the future.

# 37. Computational Complexity

The computational cost is approximately:

Ttotal≈HTstep.T_{\text{total}} \approx H T_{\text{step}}.

Thus the cost increases linearly with the number of forecast iterations.

For example, if one prediction requires approximately:

5 ms,5\text{ ms},

then:

43,200×0.005=216 seconds,43,200\times0.005 = 216\text{ seconds},

which is approximately:

3.6 minutes.3.6\text{ minutes}.

The chapter therefore argues that a day-scale forecast can be computationally practical as a batch process, although continuously running a huge open-loop forecast inside a strict 2-second acquisition cycle would not be appropriate.

# 38. Multi-Sensor Extension

The methodology can also handle multiple accelerometers.

Instead of:

St∈RM×F,S_t\in\mathbb{R}^{M\times F},

the representation becomes:

St∈RC×M×F.S_t\in\mathbb{R}^{C\times M\times F}.

For example, with CC sensors:

Sensor 1 → spectrogram ┐

Sensor 2 → spectrogram │

Sensor 3 → spectrogram ├──► Conv stem ─► ConvLSTM

Sensor 4 → spectrogram │

...                    │

Sensor C → spectrogram ┘

The convolutional stem can combine information between sensor channels.

The chapter specifically notes that this allows the recurrent memory to represent cross-sensor patterns, such as a spectral component appearing first on one sensor and subsequently on another.

# 39. What the Chapter Deliberately Does Not Use

The chapter also explains what it intentionally avoids.

### Plain CNN encoder-decoder

A CNN can be effective for one-shot spectrogram processing, but it does not naturally maintain temporal memory across multiple windows.

### Vanilla LSTM

A vanilla LSTM requires vector-like inputs, so flattening spectrograms loses their local time-frequency structure.

### Direct day-ahead prediction

A network that directly predicts a spectrogram several days ahead would have a difficult learning target because of changing operating conditions and accumulated uncertainty.

### Complex STFT prediction

The chapter predicts magnitude rather than complex phase because phase prediction introduces additional instability while magnitude captures the energy evolution of interest.

These design choices are explicitly listed in the chapter.

# 40. Practical Deployment Modes

The chapter describes three deployment scenarios.

## Mode 1 — Online Hybrid

Real measurements continuously arrive.

The system:

real data→refresh buffer→predict next window.\text{real data} \rightarrow \text{refresh buffer} \rightarrow \text{predict next window}.

This is the most closely anchored to actual machine behavior.

## Mode 2 — Gap Filling

If measurements temporarily stop:

last real context→prediction→prediction→...\text{last real context} \rightarrow \text{prediction} \rightarrow \text{prediction} \rightarrow ...

When real measurements return, the buffer is refreshed.

## Mode 3 — Scenario Forecast

The model can also be run open-loop from the current context to explore how spectral behavior might evolve.

The output must be interpreted as a forecast, rather than an actual measurement.

# 41. Complete Mathematical Formulation

The entire chapter can essentially be summarized mathematically as follows.

### Step 1: Vibration window

xt=[x[n0],...,x[n0+Nw−1]]x_t = [x[n_0],...,x[n_0+N_w-1]]

where:

Nw=fsTw.N_w=f_sT_w.

### Step 2: STFT

Xt[m,k]=∑nxt[n+mH]w[n]e−j2πkn/L.X_t[m,k] = \sum_n x_t[n+mH] w[n] e^{-j2\pi kn/L}.

### Step 3: Spectrogram

St[m,k]=log⁡(1+∣Xt[m,k]∣).S_t[m,k] = \log(1+|X_t[m,k]|).

### Step 4: Context

Ut=[St−k+1,...,St].U_t= [S_{t-k+1},...,S_t].

### Step 5: Neural prediction

S^t+1=fθ(Ut).\hat S_{t+1} = f_\theta(U_t).

### Step 6: Training

θ∗=arg⁡min⁡θ∑t∥fθ(Ut)−St+1∥F2.\theta^* = \arg\min_\theta \sum_t \left\| f_\theta(U_t)-S_{t+1} \right\|_F^2.

### Step 7: Feedback

Ut+1=[St−k+2,...,St,S^t+1].U_{t+1} = [ S_{t-k+2}, ...,S_t, \hat S_{t+1} ].

After the first iteration, predicted values gradually replace real values.

### Step 8: Long-horizon output

{S^t+1,S^t+2,...,S^t+H}.\{ \hat S_{t+1}, \hat S_{t+2}, ..., \hat S_{t+H} \}.

And:

Thorizon=HTw.T_{\text{horizon}} = HT_w.

This is the complete mathematical structure of the proposed methodology.

# 42. The Most Important Conceptual Contribution

The most important idea in this chapter is not simply using ConvLSTM.

ConvLSTM is the prediction model.

The more important system-level idea is:

one-step ConvLSTM prediction+iterative feedback=long-horizon forecasting\boxed{ \text{one-step ConvLSTM prediction} + \text{iterative feedback} = \text{long-horizon forecasting} }

The network itself remains relatively small and understandable.

The long horizon is achieved through repeated application:

fθ∘fθ∘fθ∘⋯f_\theta \circ f_\theta \circ f_\theta \circ \cdots

rather than creating a neural network with an enormous output space.

# 43. How the Whole Chapter Fits Together

The complete flow can be visualized as:

RAW VIBRATION

│

▼

┌──────────────────┐

│ 2-second window  │

│ 2400 samples     │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Overlapping      │

│ STFT windows     │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ FFT + Hann       │

│ log magnitude    │

└────────┬─────────┘

│

▼

S(t) ∈ 15×129

│

┌─────────────┼─────────────┐

│             │             │

▼             ▼             ▼

S(t-2)        S(t-1)         S(t)

│             │             │

└─────────────┼─────────────┘

│

▼

┌──────────────────┐

│ Conv Stem        │

│ local features   │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ ConvLSTM layer 1 │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ ConvLSTM layer 2 │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Conv Decoder     │

└────────┬─────────┘

│

▼

Ŝ(t+1)

│

▼

┌───────────────┐

│ FIFO feedback │

└───────┬───────┘

│

▼

Ŝ(t+2)

│

▼

Ŝ(t+3)

│

▼

...

│

▼

Ŝ(t+H)

│

▼

LONG-HORIZON FORECAST

# 44. In Simple Words

If you need to explain this chapter during a presentation or defense, I would describe it like this:

First, the continuous vibration signal is divided into two-second windows.

Second, each two-second window is converted into a time-frequency spectrogram using an overlapping STFT.

Third, several consecutive spectrograms are collected into a short context sequence.

Fourth, this sequence is given to a ConvLSTM network. ConvLSTM is used because the spectrogram contains local frequency structures and the system also needs to remember how those structures evolve over time.

Fifth, the network predicts only the next spectrogram.

Sixth, during training, the predicted spectrogram is compared with the actual next spectrogram using MSE.

Seventh, during deployment, the predicted spectrogram is placed back into the context buffer. The model then uses this new buffer to predict the following spectrogram.

Finally, this process is repeated many times. Therefore, a two-second one-step predictor can produce forecasts covering minutes, hours, or even a day.

The critical limitation is that errors can accumulate during long open-loop prediction, so the chapter recommends periodically re-anchoring the forecasting process using newly measured vibration data.

## 45. The Chapter in One Equation

If I had to reduce the entire methodology to one conceptual equation, it would be:

S^t+h=fθ(S^t+h−k,…,S^t+h−1)\boxed{ \hat S_{t+h} = f_\theta \left( \hat S_{t+h-k}, \ldots, \hat S_{t+h-1} \right) }

where the earliest terms are real measurements and, after the initial steps, increasingly become the model's own predictions.

That is what transforms short-term ConvLSTM prediction into long-horizon vibration spectrogram forecasting.

And the most important distinction to keep clear is:

ConvLSTM learns the next-window mapping;the feedback loop creates the long horizon.\boxed{ \text{ConvLSTM learns the next-window mapping;} \quad \text{the feedback loop creates the long horizon.} }

The uploaded chapter itself summarizes this architecture as vibration windows → STFT spectrograms → recent context → ConvLSTM → predicted next spectrogram → feedback → repeated long-horizon sequence.